Physics-Based Sensor Equations for Preventive Maintenance of Military Battle Tanks

Engine, Powertrain, Fluid, Structural and Acoustic Health Monitoring

This document presents the fundamental physics equations behind sensors used for preventive maintenance of military battle tanks. The framework connects physical measurements to physics-derived health indicators and AI-based fault prediction.

# 1. Vibration Sensors — Bearings, Gears and Rotating Assemblies

Accelerometers fundamentally measure mechanical acceleration:

a(t) = d²x(t)/dt²

For sinusoidal shaft vibration:

x(t) = A sin(2πft + φ)

a(t) = −(2πf)² A sin(2πft + φ)

For a rotating component with speed N in RPM:

fᵣ = N / 60

For gear-mesh frequency:

f_GMF = Nₚ Zₚ / 60

Characteristic bearing frequencies include:

BPFO = (n/2) fᵣ [1 − (d/D) cos θ]

BPFI = (n/2) fᵣ [1 + (d/D) cos θ]

Useful vibration health indicators:

RMS = √[(1/N) Σ aᵢ²]

Kurtosis = [(1/N) Σ(aᵢ − ā)⁴] / {[(1/N) Σ(aᵢ − ā)²]²}

Interpretation: increasing RMS indicates increasing vibration energy, while high kurtosis can indicate impulsive events associated with bearing or gear damage.

# 2. Temperature Sensors — Engine, Transmission and Exhaust

Thermocouples operate using the Seebeck effect:

V = ∫[Tᶜ to Tʰ] S(T) dT

For an approximately linear thermocouple response:

V ≈ S(Tₕ − T𝑐)

For an RTD, the approximate resistance-temperature relationship is:

R(T) = R₀ [1 + α(T − T₀)]

A simplified engine thermal balance is:

m cₚ (dT/dt) = Q̇_gen − Q̇_cool − Q̇_exhaust

Thermal deviations can indicate cooling-system degradation, excessive engine load, lubrication problems, combustion abnormalities, or transmission friction.

# 3. Oil Pressure Sensors

Pressure is defined by:

P = F / A

For laminar flow through a cylindrical passage, Hagen–Poiseuille's law gives:

Q = (π r⁴ / 8 μ L) ΔP

ΔP = 8 μ L Q / (π r⁴)

Oil pressure can therefore be interpreted together with engine speed, oil temperature, flow and load rather than using a fixed pressure threshold.

# 4. Oil-Debris / Metallic Particle Sensors

Inductive oil-debris sensors exploit electromagnetic properties of metallic particles.

B = μ n I

Z = R + jωL

L ≈ μ N² A / l

A metallic particle passing through the sensing region changes coil inductance or impedance, allowing particle count, size and material characteristics to be estimated.

Ṅₚ = dNₚ/dt

A rapidly increasing particle-generation rate can indicate accelerating mechanical wear.

# 5. Torque Sensors — Mechanical Load

Torque is related to tangential force by:

τ = rF

Mechanical power is:

P_mechanical = τω

ω = 2πN / 60

For a circular shaft:

τ_shear = Tr / J

γ = Tr / (JG)

Strain-based torque sensing can therefore provide mechanical load and efficiency information for engines, gearboxes and drivetrain components.

# 6. Exhaust Sensors — Combustion Health

Exhaust gases approximately follow the ideal-gas relation:

PV = nRT

P = ρ Rₛ T

Gas mass flow can be expressed as:

ṁ = ρ A v

For hydrocarbon combustion:

CₓHᵧ + (x + y/4) O₂ → xCO₂ + (y/2)H₂O

The air-fuel equivalence ratio is:

λ = (A/F)_actual / (A/F)_stoich

Exhaust temperature, pressure, oxygen concentration and flow can jointly reveal combustion abnormalities, incomplete combustion, air-fuel imbalance and exhaust restrictions.

# 7. Capacitive Fuel, Oil and Coolant Level Sensors

Capacitive level sensors use the dependence of capacitance on dielectric properties:

C = εA / d

ε = ε₀ εᵣ

As liquid height changes, the effective capacitance changes:

C = C(h)

For a simple cylindrical tank:

V = πr²h

With tank-specific calibration, the measured capacitance can be converted into liquid height and volume for fuel, oil, coolant and other fluids.

# 8. Hydraulic and Brake Fluid Pressure

Hydraulic actuation is based on Pascal's law:

P = F / A

F_out = P A_out

Fluid flow is:

Q = A v

Hydraulic power is:

P_hydraulic = P Q

Pressure, flow and temperature trends can identify leakage, pump degradation, valve faults and seal deterioration in steering, braking and turret-stabilization systems.

# 9. Fluid Degradation — Viscosity and Temperature

An empirical temperature-viscosity relationship can be represented by an Arrhenius-type model:

μ(T) = A exp[E/(RT)]

Because pressure drop depends on viscosity under laminar conditions:

ΔP ∝ μQ

Temperature, pressure and flow should therefore be interpreted together to distinguish normal thermal viscosity changes from abnormal fluid degradation.

# 10. Suspension Load Sensors — Strain and Torsion Bars

Structural stress and strain are related by:

σ = F / A

σ = Eε

ε = σ / E

A strain gauge follows approximately:

ΔR / R = GF ε

ε = ΔR / (GF R)

These relationships allow suspension loading, shock loads and structural deformation to be estimated without directly measuring force.

# 11. Torsion-Bar Physics

For a torsion bar:

τ = Tr / J

θ = TL / (JG)

T = JGθ / L

The resulting torque and twist history can be used to estimate cumulative loading and fatigue exposure.

# 12. Shock Loads

Vehicle shock loading follows Newton's second law:

F = ma

A useful vibration/shock severity indicator is RMS acceleration:

a_RMS = √[(1/T) ∫₀ᵀ a²(t) dt]

Shock histories can be accumulated to characterize terrain severity and component loading.

# 13. Acoustic Sensors — Mechanical Noise

Acoustic pressure can be represented as:

p(t) = P₀ + A sin(2πft)

Sound pressure level is:

SPL = 20 log₁₀(p_RMS / p_ref)

For air, the reference pressure is approximately:

p_ref = 20 μPa

Frequency-domain analysis uses the Fourier transform:

X(f) = ∫ x(t)e^(−j2πft) dt

Frequency analysis can reveal characteristic signatures associated with gears, bearings, pumps, shafts, valves and structural resonances.

# 14. Acoustic Emission — Structural Faults

Acoustic-emission sensors detect high-frequency elastic waves generated by crack initiation, crack propagation, friction, impact and material deformation.

∂²u/∂t² = c² ∇²u

Useful acoustic-emission features include amplitude, energy, frequency, duration and event rate.

# 15. Sensor Fusion for Predictive Maintenance

A robust tank-health system should combine multiple measurements rather than relying on a single threshold.

x(t) = [a(t), T(t), P(t), Q(t), τ(t), h(t), ε(t), p_acoustic(t), Nₚ(t)]

The combined sensor state can be mapped to a health state or fault probability:

x(t) → Health State

x(t) → P(Faultᵢ)

For remaining useful life:

RUL = f(x₁:t)

For example, simultaneously increasing oil temperature, decreasing oil pressure, increasing vibration RMS/kurtosis and increasing metallic-particle count is considerably stronger evidence of drivetrain deterioration than any one indicator alone.

# 16. Physics-Informed Predictive-Maintenance Architecture

A suitable architecture for a military battle-tank preventive-maintenance system is:

Sensors → Physics-Based Features → Digital Twin → AI → Anomaly / Fault / RUL

The physics layer can include:

Mechanics: F = ma, P = τω, σ = Eε

Fluid mechanics: P = F/A, Q = Av, ΔP–Q relationships

Thermodynamics: PV = nRT and thermal energy balance

Electromagnetics: capacitive and inductive sensing relationships

Acoustics: wave propagation and Fourier analysis

Structural mechanics: torsion, strain and fatigue loading

The AI layer can then combine raw sensor signals and physics-derived features for anomaly detection, fault classification, health-index estimation and remaining-useful-life prediction.

# 17. Sensor-to-Physics-to-Fault Mapping

| Tank Subsystem | Sensor | Physics | Key Feature | Potential Fault |
| --- | --- | --- | --- | --- |
| Engine / Powertrain | Accelerometer | a = d²x/dt² | RMS, kurtosis, FFT peaks | Bearing / gear wear |
| Engine | Temperature sensor | mcₚ dT/dt = Q̇in − Q̇out | Temperature trend | Overheating / cooling failure |
| Lubrication | Oil pressure | P = F/A; ΔP–Q | Pressure deviation | Pump / leakage / blockage |
| Lubrication | Oil-debris sensor | Electromagnetic induction | Particle count/rate | Accelerated wear |
| Powertrain | Torque sensor | P = τω; τ = Tr/J | Torque / power | Load / efficiency degradation |
| Exhaust | Gas / pressure / temperature | PV = nRT; ṁ = ρAv | EGT, pressure, λ | Combustion abnormality |
| Fuel / coolant | Capacitive level | C = εA/d | Level / volume | Leakage / depletion |
| Hydraulics | Pressure / flow | P = F/A; P_hyd = PQ | Pressure-flow relation | Pump / valve / seal fault |
| Suspension | Strain gauge | σ = Eε | Strain / load | Fatigue / overload |
| Torsion bar | Strain / angular sensor | θ = TL/JG | Torque / twist | Torsion-bar fatigue |
| Structure | Acoustic emission | Wave equation | Event rate / energy | Crack / impact / friction |

# 18. Overall Research Formulation

Physical Sensors + Physics Models + Digital Twin + AI = Predictive Maintenance

The central research proposition is to build a physics-informed digital-twin framework in which sensor measurements are converted into physically meaningful state variables, compared with expected component behavior, and then supplied to AI models for early anomaly detection, fault diagnosis and remaining-useful-life prediction.